AUTOMATED PARKING SYSTEM

ALGORITHM:

- When there is no obstacle, bot moves in a straight line.

- If the value measured by the two ultrasonic sensors on the left / right  side of the bot is less than X cm, then it understands that there is a obstacle

- Now it starts to search for parking.

- If the first parking slot if full, then it moves forward to check the next parking slot.

- This goes on until a free parking slot is detected.

- Once it identifies a free parking slot, it can get parked in  2 ways:

  - Parallel Parking

  - Vertical Parking

Parallel Parking:

- If the two ultrasonic sensors on the side have measured a value smaller than the length of the bot at an instant, the parallel parking system will operate.

- After going through the parking slot, when the distance measured by two ultrasonic sensors become equal again(when wall is detected again after passing through parking slot), the car stops

- The bot slightly moves backward and takes a 45 degree turn to the right / left.

- Now it moves backward and enter the parking area.

- While moving to the left / right, the sensors on the edges continuously measure and continue to turn left / right until the value measured by the two sensors are equal to each other. It stops when they are equal.

- The bot moves forward until the front sensor measured value is less than Y cm. When the value becomes less than Y cm, the bot stops. This marks the end of parking process.

Vertical Parking:

- If the measured value is greater than the length of the bot and the measurement lasts for ‘t' seconds,the vertical parking system will operate.

- Like in parallel parking, the bot moves till the two sensors again measure same  distance again(when wall is detected again after passing through parking slot), it stops and now it’s ready to get parked in vertical fashion.

- Then, the bot turns 90 degrees to the left / right.

- It begins to move towards the parking lot. In the meantime, the rear sensor continuously measures and if the measured value is less than Y cm, the bot stops and the parking process is completed.