AUTOMATED PARKING SYSTEM

Introduction:

In this project “Automated Parking system” we have built a low – cost bot which automatically parks in the parking space available either in parallel or vertical fashion depending upon the slot length and width. The purpose of this project is to make parking process automated and save our time and energy.

Components Required:

- Arduino Mega

- Ultrasonic sensors

- 4 wheel chasis kit

- Jumper wires

- Lithium ion batteries

- L293d motorshield

Arduino Mega:

The Arduino Mega 2560 is a microcontroller board based on the ATmega2560 (datasheet). It has 54 digital input/output pins (of which 14 can be used as PWM outputs), 16 analog inputs, 4 UARTs (hardware serial ports), a 16 MHz crystal oscillator, a USB connection, a power jack, an ICSP header, and a reset button. In this project, we have used Arduino Mega 2560 to drive the whole process of automated parking with the parking code being uploaded in it.

Ultrasonic Sensors:

An ultrasonic sensor is an electronic device that measures the distance of a target object by emitting ultrasonic sound waves, and converts the reflected sound into an electrical signal. Ultrasonic waves travel faster than the speed of audible sound (i.e. the sound that humans can hear). Ultrasonic sensors have two main components: the transmitter (which emits the sound using piezoelectric crystals) and the receiver (which encounters the sound after it has travelled to and from the target).

In order to calculate the distance between the sensor and the object, the sensor measures the time it takes between the emission of the sound by the transmitter to its contact with the receiver. The formula for this calculation is D = ½ T x C (where D is the distance, T is the time, and C is the speed of sound ~ 343 meters/second). In this project, we have used 4 ultrasonic sensors which help us in finding free parking slots and avoid accidents (since the bot stops when it finds an obstacle).

4 Wheel Chassis kit:

A four wheel chassis kit has a body, four dc motors and wheels which have to be assembled. This chassis (along with the wheels) acts as the car in our project.

L293d Motor Shield:

The L293d is a dual channel H bridge motor driver used to drive the dc motors connected to the wheels. This shield has two drivers which are capable of driving 4 dc motors. This shield is mounted over the Arduino Mega Board.

Jumper Wires:

Jumper wires are used to interconnect all the components of this project with each other and ultimately to Arduino board.

Construction:

Arduino Mega Board is placed above the Chassis – Wheel Kit. L293d Motor Shield is mounted above the Arduino Mega Board. DC Motors are connected to the wheels. Then, Connections are given using Jumper Wires.

Working Description:

Parallel parking is a method of parking a vehicle parallel to the road, in line with other parked vehicles (which is obstacles in our case). It usually requires initially driving slightly past the parking space, parallel to the obstacle in front of that space, keeping a safe distance, followed by reversing into that space.

Practically, there are 3 main steps in parallel parking.

1. Position your Car:

In our project bot, we measure the distance between and the length of the parking space and the code compares it with the length of the bot (27 cm) and if it is less than the length of the bot, then only the parallel parking is triggered.

2. Check your Mirrors:

In our case, instead of mirrors in manual parking, we adopt ultrasonic sensors (backward one) to measure the distance upto which the bot has to go and stop at the safe distance. In our Case, it is 8cm.

3. Start Backing up:

In bot's case, we measure the distance from the wall to the bot using ultrasonic sensor (The Sideward one), and the bot is Finally parked.

Generally, the Theoretical formula for minimum space required for parallel parking is given as

Min. space required = Length of Car + [√(r^2 - l^2) + (l +k)^2 - ( √(r^2- l^2)-w)^2 - l- k].

Where r= Radius of Curb Circle,

l= Length of the Car in Front of the Parking Slot,

k= Distance between Bonnet and Front Tyre,

w= Distance between the Front Tyre and the Rear Tyre.

This Completes Parellel Parking Mechanism. Now, comes another case, which is Vertical Parking. There are 3 cases in vertical parking.

Case 1:

First the bot enters into the parking area and checks for a free available slot. Once a free space is detected it checks the dimensions of the slot. If the length of the slot is greater than the length of the bot, i.e 27 cm and width of the slot is greater than the width of the bot i.e 18 cm ,then the bot gets parked by turning 90 deg towards left and by moving backward it gets parked correctly.

Case 2:

If the width of the slot is equal to the width of the bot, i.e 27 cm and length of the slot is greater than the length of the bot, i.e 18 cm, then the bot gets parked by turning 90 deg towards left and by moving backward it gets parked correctly.

Case 3:

If the width of the slot is less than the width of the bot, i.e 27 cm and length of the slot is greater than the length of the bot, i.e 18 cm. Since the dimensions does not match with the dimensions of the bot, it decides not to park and goes in search for another free slot.

This is the parking mechanism when the parking slot is at the left side. If it is present on the right side then, the opposite case happens.

There is an additional feature in our Bot. If the bot could not park in the parallel or perpendicular manner, then it will assume that there is no slot avaible to park. So, it will come back to the entrance. This will avoid unnecessary stoppage of bot in the mid-way of the road.

Conclusion:

Now let us discuss few advantages and applications of our project. We can use this feature to save our energy and time. It gets automatically parked in the parking space and helps us to avoid accidents as it has obstacle avoidance mechanism. Vehicles will also be parked in a neat manner as it gets parked according to the dimensions of the parking slot.

When vehicles become automatic, why not the parking system. Driverless cars will be a normal feature in future. So naturally, parking should also be driverless. Our project of automatic parking system is the step in that direction. We can call it as a futuristic idea but this will become a reality in the near future.